Water Quality levels for Irrigation

http://www.fao.org/docrep/003/T0234E/T0234E06.htm

| Table 21 RECOMMENDED MAXIMUM CONCENTRATIONS OF TRACE ELEMENTS IN IRRIGATION WATER1 |  |  |
| --- | --- | --- |
| Element | Recommended Maximum Concentration2 (mg/l) | Remarks |
| Al(aluminium) | 5.0 | Can cause non-productivity in acid soils (pH < 5.5), but more alkaline soils at pH > 7.0 will precipitate the ion and eliminate any toxicity. |
| As (arsenic) | 0.10 | Toxicity to plants varies widely, ranging from 12 mg/l for Sudan grass to less than 0.05 mg/l for rice. |
| Be (beryllium) | 0.10 | Toxicity to plants varies widely, ranging from 5 mg/l for kale to 0.5 mg/l for bush beans. |
| Cd (cadmium) | 0.01 | Toxic to beans, beets and turnips at concentrations as low as 0.1 mg/l in nutrient solutions. Conservative limits recommended due to its potential for accumulation in plants and soils to concentrations that may be harmful to humans. |
| Co (cobalt) | 0.05 | Toxic to tomato plants at 0.1 mg/l in nutrient solution. Tends to be inactivated by neutral and alkaline soils. |
| Cr (chromium) | 0.10 | Not generally recognized as an essential growth element. Con-servative limits recommended due to lack of knowledge on its toxicity to plants. |
| Cu (copper) | 0.20 | Toxic to a number of plants at 0.1 to 1.0 mg/l in nutrient solutions. |
| F (fluoride) | 1.0 | Inactivated by neutral and alkaline soils. |
| Fe (iron) | 5.0 | Not toxic to plants in aerated soils, but can contribute to soil acidification and loss of availability of essential phosphorus and molybdenum. Overhead sprinkling may result in unsightly deposits on plants, equipment and buildings. |
| Li (lithium) | 2.5 | Tolerated by most crops up to 5 mg/l; mobile in soil. Toxic to citrus at low concentrations (<0.075 mg/l). Acts similarly to boron. |
| Mn (manganese) | 0.20 | Toxic to a number of crops at a few-tenths to a few mg/l, but usually only in acid soils. |
| Mo (molybdenum) | 0.01 | Not toxic to plants at normal concentrations in soil and water. Can be toxic to livestock if forage is grown in soils with high concentrations of available molybdenum. |
| Ni (nickel) | 0.20 | Toxic to a number of plants at 0.5 mg/l to 1.0 mg/l; reduced toxicity at neutral or alkaline pH. |
| Pd (lead) | 5.0 | Can inhibit plant cell growth at very high concentrations. |
| Se (selenium) | 0.02 | Toxic to plants at concentrations as low as 0.025 mg/l and toxic to livestock if forage is grown in soils with relatively high levels of added selenium. An essential element to animals but in very low concentrations. |
| Sn (tin) |  |  |
| Ti (titanium) | ---- | Effectively excluded by plants; specific tolerance unknown. |
| W (tungsten) |  |  |
| V (vanadium) | 0.10 | Toxic to many plants at relatively low concentrations. |
| Zn (zinc) | 2.0 | Toxic to many plants at widely varying concentrations; reduced toxicity at pH > 6.0 and in fine textured or organic soils. |

1 Adapted from National Academy of Sciences (1972) and Pratt (1972).

2 The maximum concentration is based on a water application rate which is consistent with goodirrigation practices (10 000 m3 per hectare per year). If the water application rate greatly exceeds this, the maximum concentrations should be adjusted downward accordingly. No adjustment should be made for application rates less than 10 000 m3 per hectare per year. The values given are for water used on a continuous basis at one site.